SECTION 07 61 14.00 20

STEEL STANDING SEAM ROOFING

08/16

# PART 1   GENERAL

## 1.1   REFERENCES

The publications listed below form a part of this specification to the extent referenced.  The publications are referred to within the text by the basic designation only.

AMERICAN IRON AND STEEL INSTITUTE (AISI)

AISI SG03-3	(2002; Suppl 2001-2004; R 2008) Cold-Formed Steel Design Manual Set

ASTM INTERNATIONAL (ASTM)

ASTM A1008/A1008M	(2016) Standard Specification for Steel, Sheet, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, Solution Hardened, and Bake Hardenable

ASTM A1011/A1011M	(2017a) Standard Specification for Steel Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, and Ultra-High Strength

ASTM A36/A36M	(2014) Standard Specification for Carbon Structural Steel

ASTM A653/A653M	(2017) Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

ASTM A792/A792M	(2010) Standard Specification for Steel Sheet, 55% Aluminum-Zinc Alloy-Coated by the Hot-Dip Process

ASTM B117	(2016) Standard Practice for Operating Salt Spray (Fog) Apparatus

ASTM D1654	(2008; R 2016; E 2017) Standard Test Method for Evaluation of Painted or Coated Specimens Subjected to Corrosive Environments

ASTM D2244	(2016) Standard Practice for Calculation of Color Tolerances and Color Differences from Instrumentally Measured Color Coordinates

ASTM D2247	(2015) Testing Water Resistance of Coatings in 100% Relative Humidity

ASTM D226/D226M	(2017) Standard Specification for Asphalt-Saturated Organic Felt Used in Roofing and Waterproofing

ASTM D4214	(2007; R 2015) Standard Test Method for Evaluating the Degree of Chalking of Exterior Paint Films

ASTM D522/D522M	(2014) Mandrel Bend Test of Attached Organic Coatings

ASTM D523	(2014; R 2018) Standard Test Method for Specular Gloss

ASTM D714	(2002; R 2017) Standard Test Method for Evaluating Degree of Blistering of Paints

ASTM D968	(2017) Standard Test Methods for Abrasion Resistance of Organic Coatings by Falling Abrasive

ASTM E1592	(2005; R 2012) Structural Performance of Sheet Metal Roof and Siding Systems by Uniform Static Air Pressure Difference

ASTM E84	(2018) Standard Test Method for Surface Burning Characteristics of Building Materials

ASTM G152	(2013) Operating Open Flame Carbon Arc Light Apparatus for Exposure of Nonmetallic Materials

ASTM G153	(2013) Operating Enclosed Carbon Arc Light Apparatus for Exposure of Nonmetallic Materials

SHEET METAL AND AIR CONDITIONING CONTRACTORS' NATIONAL ASSOCIATION (SMACNA)

SMACNA 1793	(2012) Architectural Sheet Metal Manual, 7th Edition

U.S. DEPARTMENT OF ENERGY (DOE)

Energy Star	(1992; R 2006) Energy Star Energy Efficiency Labeling System (FEMP)

## 1.2   DEFINITIONS

### 1.2.1   Field-Formed Seam

Seams of panels so configured that when adjacent sheets are installed the seam is sealed utilizing mechanical or hand seamers.  Crimped (45 degree bend), roll formed (180 degree bend), double roll formed (2 - 180 degree bends), and roll and lock systems are types of field-formed seam systems.

### 1.2.2   Snap Together Seam

Panels so configured that the male and female portions of the seam interlock through the application of foot pressure or tamping with a mallet.  Snap-on cap configurations are a type of snap together system.

### 1.2.3   Pre-Formed

Formed to the final, less field-formed seam, profile and configuration in the factory.

### 1.2.4   Field-Formed

Formed to the final, less field-formed seam, profile and configuration at the site of work prior to installation.

### 1.2.5   Roofing System

The roofing system is defined as the assembly of roofing components, including roofing panels, flashing, fasteners, and accessories which, when assembled properly result in a watertight installation.

### 1.2.6   SSMRS

Standing Seam Metal Roof System (SSMRS) is abbreviation of the entire roof system specified herein with all components and parts coming from a single manufacturer's system.

## 1.3   SYSTEM DESCRIPTION

### 1.3.1   Design Requirements

a.  Panels must be continuous lengths up to manufacturer's standard longest lengths, with no joints or seams, except where indicated or specified.  Ribs of adjoining sheets must be in continuous contact from eave to ridge.  Individual panels of snap together type systems must be removable for replacement of damaged material.

b.  There must be no exposed or penetrating fasteners except where shown on approved shop drawings.  Fasteners into steel must be stainless steel, zinc cast head, or cadmium plated steel screws inserted into predrilled holes.  There must be a minimum of two fasteners per clip.  Single fasteners will be allowed when supporting structural members are prepunched or predrilled.

c.  Snap together type systems must have a capillary break and a positive side lap locking device.  Field-formed seam type systems must be mechanically locked closed by the manufacturer's locking tool.  The seam must include a continuous factory applied sealant when required by the manufacturer to withstand the wind loads specified.

d.  Roof panel anchor clips must be concealed and designed to allow for longitudinal thermal movement of the panels, except where specific fixed points are indicated.  Provide for lateral thermal movement in panel configuration or with clips designed for lateral and longitudinal movement.

### 1.3.2   Design Conditions

Design the system to resist positive and negative loads specified herein in accordance with the AISI SG03-3.  Panels must support walking loads without permanent distortion or telegraphing of the structural supports.

#### 1.3.2.1   Wind Uplift

Compute and apply the design uplift pressures for the roof system using a basic wind speed of per local building code miles per hour (mph).  Roof system and attachments must resist the following wind loads, in pounds per square foot (psf):

|  | Negative |
| --- | --- |
| a. At eaves | per code |
| b. At rakes | per code |
| c. At ridge | per code |
| d. At building corners | per code |
| e. At central areas | per code |

The design uplift force for each connection assembly must be that pressure given for the area under consideration, multiplied by the tributary load area of the connection assembly, and multiplied by the appropriate factor of safety, as follows:

a.  Single fastener in a connection:  3.0

b.  Two or more fasteners in each connection:  2.25

#### 1.3.2.2   Roof Live Loads

Loads must be applied on the horizontal projection of the roof structure.  The minimum roof design live load must be 20 psf.

#### 1.3.2.3   Thermal Movement

System must be capable of withstanding thermal movement based on a temperature range of 10 degrees F below low temp for project location per UFC 3-400-02 " Design Engineering Weather Data" degrees F and

#### 1.3.2.4   Deflection

Panels must be capable of supporting design loads between unsupported spans with deflection of not greater than L/180 of the span.

### 1.3.3   Structural Performance

The structural performance test methods and requirements of the Standing Seam Roofing Systems (SSRS) must be in accordance with ASTM E1592.

## 1.4   SUBMITTALS

Government approval may be required on any of the following items.  Items requiring submittals will be listed in the submittal register.  Submit the required items in accordance with Section 01 33 00 SUBMITTAL PROCEDURES:

SD-02 Shop Drawings

Roofing;

SD-03 Product Data

Roofing Panels;

Energy Star Label for Steel Roofing Product;

Recycled Content for Steel Roofing Product;

Heat Island Reduction;

Attachment Clips

Closures

Accessories

Fasteners

Sealants

Insulation, including Joint Sealing Measures for Vapor Barrier Facing

Sample Warranty Certificate;

Submit for materials to be provided.  Submit data sufficient to indicate conformance to specified requirements.

SD-04 Samples

Roofing Panel

Submit a 12 inch long by full width section of typical panel.

For color selection, submit 2 by 4 inch metal samples in color, finish and texture selected.  When colors are not indicated, submit samples of not less than six different manufacturer's standard colors for selection.

Accessories

Submit each type of accessory item used in the project including, but not limited to each type of anchor clip, closure, fastener, and leg clamp.

Sealants

Intermediate Support Section

Submit full size samples of each intermediate support section, 12 inches long.

SD-05 Design Data

Design Calculations

SD-06 Test Reports

Field Inspection;

Submit manufacturer's technical representative's field inspection reports as specified in paragraph MANUFACTURER'S FIELD INSPECTION.

Structural Performance Tests

Finish Tests

SD-07 Certificates

Manufacturer's Technical Representative's Qualifications

Statement of Installer's Qualifications

Submit documentation from roofing manufacturer proving the manufacturer's technical representative meets below specified requirements.  Include name, address, telephone number, and experience record.

Submit documentation proving the installer is factory-trained, has the specified experience, and authorized by the manufacturer to install the products specified.

Coil Stock Compatibility;

Provide certification of coil compatibility with roll forming machinery to be used for forming panels without warping, waviness, and rippling not part of panel profile; to be done without damage, abrasion or marking of finish coating.

SD-08 Manufacturer's Instructions

Installation Manual;

Submit manufacturers printed installation manual, instructions, and standard details.

SD-11 Closeout Submittals

Information Card

For each roofing installation, submit a typewritten card or photoengraved aluminum card containing the information listed on Form 1 located at the end of this section.

Warranty

## 1.5   DESIGN CALCULATIONS

Provide design calculations prepared by a professional engineer specializing in structural engineering verifying that system supplied and any additional framing meets design load criteria indicated.  Coordinate calculations with manufacturer's test results.  Include calculations for:

Wind load uplift design pressure at roof locations specified in paragraph WIND UPLIFT.

Clip spacing and allowable load per clip.

Fastening of clips to structure or intermediate supports.

Intermediate support spacing and framing and fastening to structure when required.

Allowable panel span at anchorage spacing indicated.

Safety factor used in design loading.

Governing code requirements or criteria.

Edge and termination details.

## 1.6   QUALITY ASSURANCE

### 1.6.1   Preroofing Conference

After submittals are received and approved but before roofing and insulation work, including associated work, is preformed, the Contractor must hold a preroofing conference to review the following:

a.  The drawings and specifications

b.  Procedure for on-site inspection and acceptance of the roofing substrate and pertinent structural details relating to the roofing system

c.  Contractor's plan for coordination of the work of the various trades involved in providing the roofing system and other components secured to the roofing

d.  Safety requirements

The preroofing conference must be attended by the Contractor and personnel directly responsible for the roofing and insulation installation, mechanical and electrical work, and the roofing manufacturer's technical representative.  Conflicts among those attending the preroofing conference must be resolved and confirmed in writing before roofing work, including associated work, is begun. Prepare written minutes of the preroofing conference and submit to the Contracting Officer.

### 1.6.2   Manufacturer

The SSMRS must be the product of a metal roofing industry - recognized manufacturer who has been in the practice of manufacturing SSMRS for a period of not less than 5 years and who has been involved in at least 5 projects similar in size and complexity to this project.

### 1.6.3   Manufacturer's Technical Representative

The representative must have authorization from manufacturer to approve field changes and be thoroughly familiar with the products and with installations in the geographical area where construction will take place.  The manufacturer's representative must be an employee of the manufacturer with at least 5 years experience in installing the roof system.  The representative must be available to perform field inspections and attend meetings as required herein, and as requested by the Contracting Officer.

### 1.6.4   Installer's Qualifications

The roofing system installer must be factory-trained, approved by the steel roofing system manufacturer to install the system, and must have a minimum of three years experience as an approved applicator with that manufacturer.  The applicator must have applied five installations of similar size and scope as this project within the previous 3 years.

### 1.6.5   Single Source

Roofing panels, clips, closures, and other accessories must be standard products of the same manufacturer; must be the latest design by the manufacturer; and must have been designed by the manufacturer to operate as a complete system for the intended use.

### 1.6.6   Laboratory Tests For Panel Finish

The term "appearance of base metal" refers to the metal coating on steel.  Panels must meet the following test requirements:

a.  Formability Test:  When subjected to a 180 degree bend over a 1/8 inch diameter mandrel in accordance with ASTM D522/D522M, exterior coating film may show only slight microchecking and no loss of adhesion.

b.  Accelerated Weathering Test:  Withstand a weathering test for a minimum of 2000 hours in accordance with ASTM G152 and ASTM G153, Method 1 without cracking, peeling, blistering, loss of adhesion of the protective coating, or corrosion of the base metal.  Protective coating that can be readily removed from the base metal with a penknife blade or similar instrument will be considered to indicate loss of adhesion.

c.  Chalking Resistance:  After the 2000-hour weatherometer test, exterior coating may not chalk greater than No. 8 rating when measured in accordance with ASTM D4214 test procedures.

d.  Color Change Test:

After the 2000 -hour weatherometer test, exterior coating color change must not exceed 2 NBS units when measured in accordance with ASTM D2244 test procedure.

e.  Salt Spray Test:  Withstand a salt spray test for a minimum of 1000 hours in accordance with ASTM B117, including the scribe requirement in the test.  Immediately upon removal of the panel from the test, the coating must receive a rating of 8, few blisters in field as determined by ASTM D714; and an average rating of 6, 1/8 inch failure at scribe, as determined by ASTM D1654.  Rating Schedule No. 1.

f.  Abrasion Resistance Test for Color Coating:  When subjected to the falling sand test in accordance with ASTM D968, coating system must withstand a minimum of 50 liters of sand per mil thickness before appearance of base metal.

g.  Humidity Test:  When subjected to a humidity cabinet test in accordance with ASTM D2247 for 1000 hours, a scored panel must show no signs of blistering, cracking, creepage, or corrosion.

h.  Gloss Test:  The gloss of the finish must be 30 plus or minus 5 at an angle of 60 degrees, when measured in accordance with ASTM D523.

i.  Glare Resistance Test:

Surfaces of panels that will be exposed to the exterior must have a specular reflectance of not more than 10 when measured in accordance with ASTM D523 at an angle of 85 degrees.  Specular reflectance may be obtained with striations or embossing.  Requirements specified under FORMABILITY TEST will be waived if necessary to conform to this requirement.

### 1.6.7   Shop Drawing Requirements

Submit roofing drawings to supplement the instructions and diagrams.  Include design and erection drawings containing an isometric view of the roof showing the design uplift pressures and dimensions of edge, ridge and corner zones; and show typical and special conditions including flashings, materials and thickness, dimensions, fixing lines, anchoring methods, sealant locations, sealant tape locations, fastener layout, sizes, and spacing, terminations, penetrations, attachments, and provisions for thermal movement.  Details of installation must be in accordance with the manufacturer's Standard Instructions and details or the SMACNA 1793.  Prior to submitting shop drawings, have drawings reviewed and approved by the manufacturer's technical engineering department.

## 1.7   WARRANTY

Furnish manufacturer's no-dollar-limit materials and workmanship warranty for the roofing system.  The warranty period must be not less than 20 years from the date of Government acceptance of the work.  The warranty must be issued directly to the Government.  The warranty must provide that if within the warranty period the metal roofing system becomes non-watertight or shows evidence of corrosion, perforation, rupture or excess weathering due to deterioration of the roofing system resulting from defective materials or installed workmanship the repair or replacement of the defective materials and correction of the defective workmanship must be the responsibility of the roofing system manufacturer.  Repairs that become necessary because of defective materials and workmanship while roofing is under warranty must be performed within 7 days after notification, unless additional time is approved by the Contracting Officer.  Failure to perform repairs within the specified period of time will constitute grounds for having the repairs performed by others and the cost billed to the manufacturer.  In addition, provide a 2 year contractor installation warranty.

## 1.8   DELIVERY, STORAGE AND HANDLING

Deliver, store, and handle preformed panels, bulk roofing products and other manufactured items in a manner to prevent damage or deformation.

### 1.8.1   Delivery

Provide adequate packaging to protect materials during shipment.  Crated materials must not be uncrated until ready for use, except for inspection. Immediately upon arrival of materials at the jobsite, inspect materials for damage, dampness, and staining.  Replace damaged or permanently stained materials that cannot be restored to like-new condition with satisfactory material.  If materials are wet, remove the moisture and re-stack and protect the panels until used.

### 1.8.2   Storage

Stack materials on platforms or pallets and cover with tarpaulins or other suitable weathertight covering which prevents water trapping or condensation.  Store materials so that water which might have accumulated during transit or storage will drain off.  Do not store the panels in contact with materials that might cause staining, such as mud, lime, cement, fresh concrete or chemicals.  Protect stored panels from wind damage.

### 1.8.3   Handling

Handle material carefully to avoid damage to surfaces, edges and ends.

# PART 2   PRODUCTS

## 2.1   ROOFING PANELS

Provide panels with interlocking ribs for securing adjacent sheets and with concealed clip fastening system for securing the roof covering to structural framing members.  Fasteners must not penetrate the panels except at the ridge, eave, rakes, penetrations, and end laps.  Backing plates and ends of panels at end laps must be predrilled or prepunched.  Factory prepare ends of panels to be lapped by trimming part of seam, die-setting, or swaging ends of panels.  Individual sheets must be sufficiently long to cover the entire length of any unbroken roof slope when such slope is 30 feet or less.  Provide panels that extend over two or more spans when length of run exceeds 30 feet.  Obtain Contracting Officer (KO) approval for sheets longer than 30 feet before submitting shop drawings.  Sheets must provide not less than 12 inches of coverage (width) in place.  Provide panels with a minimum corrugation height of 2.25 inches (nominal).  Make provisions for expansion and contraction at either ridge or eave, consistent with the type of system to be used.  Form panels from coil stock without warping, waviness or ripples not part of the panel profile, and free of damage to the finish coating system.

Provide steel roofing product that is Energy Star labeled.  Provide data identifying Energy Star label for steel roofing product.  Provide solar reflectance product with an initial solar reflectance greater than or equal to 0.25 and a solar reflectance greater than or equal to 0.15 three years after installation under normal conditions.  Provide emittance and reflectance percentages, solar reflectance index values, and slopes , to meet sustainable third party certification requirements for Heat Island Reduction.

### 2.1.1   Material

Zinc-coated steel conforming to ASTM A653/A653M, G90 coating designation or aluminum-zinc alloy coated steel conforming to ASTM A792/A792M, AZ 55 coating.  Provide material with a minimum thickness of 0.023 inch thick (24 gage) minimum except when mid field of roof is subject to design wind uplift pressures of 60 psf or greater, entire roof system must have a minimum thickness of 0.030 inch (22 gage).  Steel roofing materials must contain a minimum of 30 percent total recycled content.  Provide data identifying percentage of recycled content for steel roofing product.  Prior to shipment, treat mill finish panels with a passivating chemical and oil to inhibit the formation of oxide corrosion products.  Dry, retreat, and re-oil panels that have become wet during shipment or storage but have not started to oxidize.

### 2.1.2   Texture

Smooth with raised intermediate ribs for added stiffness.

### 2.1.3   Finish

Factory color finish.

#### 2.1.3.1   Factory Color Finish

Provide factory applied, thermally cured coating to exterior and interior of metal roof and wall panels and metal accessories.  Provide exterior finish top coat of 70 percent resin polyvinylidene fluoride with not less than 0.8 mil dry film thickness.  Provide exterior primer standard with panel manufacturer with not less than 0.8 mil dry film thickness.  Interior finish must consist of the same coating and dry film thickness as the exterior coating.  Provide exterior and interior coating meeting test requirements specified below.  Tests must have been performed on the same factory finish and thickness provided.  Provide clear factory edge coating on all factory cut or unfinished edges.

## 2.2   INTERMEDIATE SUPPORTS

Fabricate panel subgirts, subpurlins, T-bars, Z-bars and tracks from galvanized steel conforming to ASTM A653/A653M, G90, Grade D ( 16 gage and heavier), Grade A ( 18 gage and lighter); or steel conforming to ASTM A36/A36M, ASTM A1011/A1011M , or ASTM A1008/A1008M prime painted with zinc-rich primer.  Size, shape, thickness and capacity as required to meet the load, insulation thickness and deflection criteria specified.

## 2.3   ATTACHMENT CLIPS

Fabricate clips from ASTM A1011/A1011M, or ASTM A1008/A1008M steel hot-dip galvanized in accordance with ASTM A653/A653M, G 90, or Series 300 stainless steel.  Size, shape, thickness and capacity as required to meet the load, insulation thickness and deflection criteria specified.

## 2.4   ACCESSORIES

Sheet metal flashings, gutters, downspouts, trim, moldings, closure strips, pre-formed crickets, caps, equipment curbs, and other similar sheet metal accessories used in conjunction with preformed metal panels must be of the same material as used for the panels.  Provide metal accessories with a factory color finish to match the roofing panels, except that such items which will be concealed after installation may be provided without the finish if they are stainless steel.  Metal must be of a thickness not less than that used for the panels.  Thermal spacer blocks and other thermal barriers at concealed clip fasteners must be as recommended by the manufacturer except that wood spacer blocks are not allowed.

### 2.4.1   Closures

#### 2.4.1.1   Rib Closures

Corrosion resisting steel, closed-cell or solid-cell synthetic rubber, neoprene or polyvinyl chloride pre-molded to match configuration of rib opening.  Material for closures must not absorb water.

#### 2.4.1.2   Ridge Closures

Metal-clad foam or metal closure with foam secondary closure matching panel configuration for installation on surface of roof panel between panel ribs at ridge and headwall roof panel flashing conditions and terminations.  Foam material must not absorb water.

### 2.4.2   Fasteners

Zinc-coated steel, corrosion resisting steel, zinc cast head, or nylon capped steel, type and size specified below or as otherwise approved for the applicable requirements.  Design the fastening system to withstand the design loads specified.  Exposed fasteners must be gasketed or have gasketed washers on the exterior side of the covering to waterproof the penetration.  Washer material must be compatible with the covering; have a minimum diameter of 3/8 inch for structural connections; and gasketed portion of fasteners or washers must be neoprene or other equally durable elastomeric material approximately 1/8 inch thick.

#### 2.4.2.1   Screws

Not smaller than No. 14 diameter if self-tapping type and not smaller than No. 12 diameter if self-drilling and self-tapping.

#### 2.4.2.2   Bolts

Not smaller than 1/4 inch diameter, shouldered or plain shank as required, with proper nuts.

#### 2.4.2.3   Automatic End-Welded Studs

Automatic end-welded studs must be shouldered type with a shank diameter of not smaller than 3/16 inch and cap or nut for holding covering against the shoulder.

#### 2.4.2.4   Explosive Driven Fasteners

Fasteners for use with explosive actuated tools must have a shank diameter of not smaller than 0.145 inch with a shank length of not smaller than 1/2 inch for fastening to steel and not smaller than 1 inch for fastening to concrete.

#### 2.4.2.5   Rivets

Blind rivets must be stainless steel with 1/8 inch nominal diameter shank. Rivets must be threaded stem type if used for other than the fastening of trim.  Rivets with hollow stems must have closed ends.

### 2.4.3   Sealants

Elastomeric type containing no oil or asphalt.  Exposed sealant must cure to a rubberlike consistency.  Concealed sealant must be the non-hardening type.  Seam sealant must be factory-applied, non-skinning, non-drying, and must conform to the roofing manufacturer's recommendations.  Silicone-based sealants must not be used in contact with finished metal panels and components unless approved otherwise by the Contracting Officer.

### 2.4.4   GASKETS AND INSULATING COMPOUNDS

Nonabsorptive and suitable for insulating contact points of incompatible materials.  Insulating compounds must be nonrunning after drying.

## 2.5   THERMAL INSULATION

Flexible blanket, rigid, or semi-rigid faced with a flexible vapor retarder.  Insulation and facing must have a flame-spread rating of 50 or less in accordance with ASTM E84.  Vapor retarder facing must have a permeance rating of 0.05 perm or less.  Provide a thermal resistance "R" value of as indicated or more.  Exposed insulation must have a white nondusting and nonshedding finish.  Facings and finishes must be factory-applied.

## 2.6   UNDERLAYMENT FOR WOOD SUBSTRATES

ASTM D226/D226M, Type I perforated, covered by water-resistant rosin sized building paper.

## 2.7   LINER PANELS

Fabricate liner panels of the same material as roof panels, and formed or patterned to prevent waviness and distortion.  Liner panels must have a factory applied, one mil thick minimum painted coating on the inside face and a prime coat on the liner side.

# PART 3   EXECUTION

Do not install building construction materials that show visible evidence of biological growth.

## 3.1   EXAMINATION

Examine surfaces to receive standing seam metal roofing and flashing.  Ensure that surfaces are plumb and true, clean, even, smooth, as dry and free from defects and projections which might affect the installation.

## 3.2   PROTECTION FROM CONTACT WITH DISSIMILAR MATERIALS

### 3.2.1   Cementitious Materials

Paint metal surfaces which will be in contact with mortar, concrete, or other masonry materials with one coat of alkali-resistant coating such as heavy-bodied bituminous paint.

### 3.2.2   Contact with Wood

Where metal will be in contact with wood or other absorbent material subject to wetting, seal joints with sealing compound and apply one coat of heavy-bodied bituminous paint.

## 3.3   INSTALLATION

Install in accordance with the approved manufacturer's erection instructions, shop drawings, and diagrams.  Panels must be in full and firm contact with attachment clips.  Where prefinished panels are cut in the field, or where any of the factory applied coverings or coatings are abraded or damaged in handling or installation, they must, after necessary repairs have been made with material of the same color as the weather coating, be approved before being installed.  Seal completely openings through panels.  Correct defects or errors in the materials.  Replace materials which cannot be corrected in an approved manner with nondefective materials.  Provide molded closure strips where indicated and where necessary to provide weathertight construction.  Use shims as required to ensure attachment clip line is true.  Use a spacing gage at each row of panels to ensure that panel width is not stretched or shortened.  Provide one layer of asphalt-saturated felt placed perpendicular to roof slope, covered by one layer of rosin-sized building paper placed parallel to roof slope with side laps down slope and attached with roofing nails.  Overlap side and end laps 3 inches, offset seams in building paper with seams in felt.

### 3.3.1   Roof Panels

Apply roofing panels with the standing seams parallel to the slope of the roof.  Provide roofing panels in longest practical lengths from ridge to eaves (top to eaves on shed roofs), with no transverse joints except at the junction of ventilators, curbs, skylights, chimneys, and similar openings.  Install flashing to assure positive water drainage away from roof penetrations.  Locate panel end laps such that fasteners do not engage supports or otherwise restrain the longitudinal thermal movement of panels.  Form field-formed seam type system seams in the field with an automatic mechanical seamer approved by the manufacturer.  Attach panels to the structure with concealed clips incorporated into panel seams.  Clip attachment must allow roof to move independently of the structure, except at fixed points as indicated.

### 3.3.2   Insulation Installation

Install between covering and supporting members to present a neat appearance.  Fold and staple and tape seams unless approved otherwise by the Contracting Officer.

#### 3.3.2.1   Rigid or Semi-Rigid Insulation

Install in areas where insulation is exposed to view.  Fasten securely without loose joints or unsightly sags.

#### 3.3.2.2   Blanket Insulation

May be used in concealed locations.  Lap facing at joints and fasten in a manner that will provide tight joints.

### 3.3.3   Flashings

Provide flashing, related closures and accessories as indicated and as necessary to provide a weathertight installation.  Install flashing to ensure positive water drainage away from roof penetrations.  Flash and seal the roof at the ridge, eaves and rakes, and projections through the roof.  Place closure strips, flashing, and sealing material in an approved manner that will assure complete weathertightness.  Details of installation which are not indicated must be in accordance with the SMACNA 1793, panel manufacturer's approved printed instructions and details, or the approved shop drawings.  Allow for expansion and contraction of flashing.

### 3.3.4   Flashing Fasteners

Fastener spacings must be in accordance with the panel manufacturer's recommendations and as necessary to withstand the design loads indicated.  Install fasteners in roof valleys as recommended by the manufacturer of the panels.  Install fasteners in straight lines within a tolerance of 1/2 inch in the length of a bay. Drive exposed penetrating type fasteners normal to the surface and to a uniform depth to seat gasketed washers properly and drive so as not to damage factory applied coating.  Exercise extreme care in drilling pilot holes for fastenings to keep drills perpendicular and centered.  Do not drill through sealant tape.  After drilling, remove metal filings and burrs from holes prior to installing fasteners and washers.  Torque used in applying fasteners must not exceed that recommended by the manufacturer.  Remove panels deformed or otherwise damaged by over-torqued fastenings, and provide new panels.

### 3.3.5   Rib and Ridge Closure/Closure Strips

Set closure/closure strips in joint sealant material and apply sealant to mating surfaces prior to adding panel.

## 3.4   PROTECTION OF APPLIED ROOFING

Do not permit storing, walking, wheeling, and trucking directly on applied roofing materials.  Provide temporary walkways, runways, and platforms of smooth clean boards or planks as necessary to avoid damage to applied roofing materials, and to distribute weight to conform to indicated live load limits of roof construction.

## 3.5   CLEANING

Clean exposed sheet metal work at completion of installation.  Remove metal shavings, filings, nails, bolts, and wires from roofs.  Remove grease and oil films, excess sealants, handling marks, contamination from steel wool, fittings and drilling debris and scrub the work clean.  Exposed metal surfaces must be free of dents, creases, waves, scratch marks, solder or weld marks and damage to the finish coating.

## 3.6   MANUFACTURER'S FIELD INSPECTION

Manufacturer's technical representative must visit the site as necessary during the installation process to assure panels, flashings, and other components are being installed in a satisfactory manner.  Manufacturer's technical representative must perform a field inspection during the first 20 squares of roof panel installation and at substantial completion prior to issuance of warranty, as a minimum, and as otherwise requested by the Contracting Officer.  Additional inspections must not exceed one for 100 squares of total roof area with the exception that follow-up inspections of previously noted deficiencies or application errors must be performed as requested by the Contracting Officer.  Each inspection visit must include a review of the entire installation to date.  After each inspection, submit a report, signed by the manufacturer's technical representative, to the Contracting Officer noting the overall quality of work, deficiencies and any other concerns, and recommended corrective actions in detail.  Notify Contracting Officer a minimum of 2 working days prior to site visit by manufacturer's technical representative.

## 3.7   COMPLETED WORK

Completed work must be plumb and true without oil canning, dents, ripples, abrasion, rust, staining, or other damage detrimental to the performance or aesthetics of the completed roof assembly.

## 3.8   INFORMATION CARD

For each roof, provide a typewritten card, laminated in plastic and framed for interior display or a photoengraved 0.032 inch thick aluminum card for exterior display.  Card to be 8 1/2 by 11 inches minimum and contain the information listed on Form 1 at end of this section.  Install card near point of access to roof, or where indicated.  .

## 3.9   FORM ONE

FORM 1  -  PREFORMED STEEL STANDING SEAM ROOFING SYSTEM COMPONENTS

1.  Contract Number:

2.  Building Number & Location:

3.  NAVFAC Specification Number:

4.  Deck/Substrate Type:

5.  Slopes of Deck/Roof Structure:

6.  Insulation Type & Thickness:

7.  Insulation Manufacturer:

8.  Vapor Retarder:    ( )Yes    ( )No

9.  Vapor Retarder Type:

10. Preformed Steel Standing Seam Roofing Description:

a.  Manufacturer (Name, Address, & Phone No.):

b.  Product Name:              c.  Width:            d.  Gage:

e.  Base Metal:                f.  Method of Attachment:

11. Repair of Color Coating:

a.  Coating Manufacturer (Name, Address & Phone No.):

b.  Product Name:

c.  Surface Preparation:

d.  Recoating Formula:

e.  Application Method:

12. Statement of Compliance or Exception:_________________________________

__________________________________________________________________________

__________________________________________________________________________

13. Date Roof Completed:

14. Warranty Period:  From_______________  To_______________

15. Roofing Contractor (Name & Address):

16. Prime Contractor (Name & Address):

Contractor's Signature _________________________  Date:

Inspector's Signature  _________________________  Date:

-- End of Section --